The df files contain continuous, 1-Hz water column

readings for Scripps Pier and Kings Bay from

12/26/2004 00:00:00 to 12/31/2004 23:59:59 UTC.

More sensor details are given below.

There are missing values in the Kings Bay file; these

are recorded as -1.0 in df08301. Neither of the files

has been detrended for tides. The Kings Bay file also

contains some reception-related spikes which have not

been removed.

The sp links return CDIP spectral files concatenated

together, including the file headers.

--------------------------------------------------------------------------------

Name: SCRIPPS PIER, LA JOLLA CA

Station: 07301

Channel description: SINGLE POINT

Deployment latitude: 32 52.000' N

Deployment longitude: 117 15.400' W

Water depth(cm):    680

Sensor elevation from sea floor(cm):    173

Data type: Water column (cm)

Gauge type: Digital Paros pressure sensor

Sample rate(Hz): 1.000

Field software version: sci_acq

Field station type: sun

Method of analysis: CDIP

Calibration factor: 1.0000

Offset:      0.00

Radio modem: no

Energy basin processing: no

Surge filter processing: no

Number of records: 518400

Start time: 20041226000000 UTC

End time:   20041231235959 UTC